TABLE 4-1  AREA AND HEIGHT LIMITATIONS FOR UNSPRINKLERED BUILDINGS AND SPACES

| Occupancy Group |  | NON-COMBUSTIBLE CONSTRUCTION GROUP I |  |  |  |  | COMBUSTIBLE CONSTRUCTION GROUP II |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class IA | Class IB | Class IC | Class ID | Class IE | Class IIA | Class IIB | Class IIC | Clss IID | Class IIE |
| # RESIDENTIAL # J-2 When Constructed Subject to the Requirements of §27-954(t) or Altered Subject to the Requirements of §27-123.2 | # Area | N.P. | N.P. | N.P. | N.P. | N.P. | N.P. | N.P. | N.P. | N.P. | N.P. |
|  | # Height |  |  |  |  |  |  |  |  |  |  |
| RESIDENTIAL J-2 When Not Subject to the Requirements of §27-954(t) or Altered Subject to the Requirements of §27-123.2 | Area | N.L. | N.L. | N.L. 75’–0” | 17,500 75’-0” (6) | N.P. | 10,000 75’-0”(6) | 10,000 75’-0”(6) | 5,600 40’-0”(3) | N.P. | N.P. |
|  | Height |  |  |  |  |  |  |  |  |  |  |