Int. No. 1448

By Council Members Williams and Torres

..Title

A Local Law to amend the New York city building code, in relation to requiring site safety plans and a monitoring program for buildings more than four stories in height

..Body

Be it enacted by the Council as follows:

Section 1. The title of section BC 3310 of the New York city building code is amended to read as follows:

SECTION BC 3310

REQUIREMENTS FOR THE CONSTRUCTION OR

DEMOLITON OF [MAJOR] CERTAIN BUILDINGS

§ 2. Section 3310.1 of the New York city building code is amended to read as follows:

3310.1 Scope. This section shall apply to:

- The construction of a new (i) major building or (ii) building more than four stories or 40 feet (12 192 mm) in height;

- The vertical or horizontal enlargement of (i) a major building or (ii) a building more than four stories or 40 feet (12 192 mm) in height;

- The full or partial demolition of (i) a major building or (ii) a building more than four stories or 40 feet (12 192 mm) in height;

- The alteration, maintenance, or repair of a façade of a major building, provided the                  building is more than 14 stories or 200 feet (60 960 mm) in height and also provided the façade work requires a sidewalk shed to be installed; and

- Any construction or demolition work, including the alteration, maintenance, or           repair of a façade, in a building so designated by the commissioner.

Exception: The requirements of this section shall not apply to partial demolition operations limited to the interior components of a [major] building, provided no mechanical demolition equipment, other than handheld devices, [are] is used.

§ 3. Section 3310.5 of the New York city building is amended to read as follows:

3310.5 Site safety manager or coordinator to be designated. One or more site safety managers shall be designated, as necessary, to ensure compliance with the site safety plan and all site safety requirements as specified in this chapter. Such site safety manager or managers shall be designated by the owner, agent, construction manager, or general contractor. All such entities shall agree to designate one such site safety manager as the primary site safety manager, or where there is only one site safety manager, such manager shall automatically be designated as the primary site safety manager. Such site safety manager(s) shall be shall be certified by the department in accordance with Article 402 of Chapter 4 of Title 28 of the Administrative Code.

Exception: One or more site safety coordinators, certified by the department in accordance with the requirements of Article 403 of Chapter 4 of Title 28 of the Administrative Code, may be designated in lieu of a site safety manager for the construction, vertical or horizontal enlargement, or full or partial demolition of a [major] building, provided such building:

1.	Is less than 15 stories or 200 feet (60 960 mm) in height; and

2.	Has a building footprint of 100,000 square feet (30 480 m2) or less.

§ 4. This local law shall take effect 180 days after it becomes law, except that the commissioner of buildings may take such measures as are necessary for its implementation, including the promulgation of rules, before such effective date.

GP

LS 6762

12/22/16 2:35PM