Int. No. 998

By Council Members Gerson, The Speaker (Council Member Quinn), Crowley, Ulrich, White Jr., Koppell, Gennaro, Brewer, Fidler, Gentile, Gonzalez, James, Liu, Nelson, Reyna, Seabrook, Stewart and Weprin (in conjunction with the Mayor)

..Title

A Local Law to amend the administrative code of the city of New York, in relation to the simultaneous performance of demolition work and  asbestos activities within the same building.

..Body

Be it enacted by the Council as follows:

Section 1. Section 24-146.1 of the administrative code of the city of New York is amended by adding a new subdivision o to read as follows:

(o) (1) No asbestos abatement activities shall be performed in a building simultaneously with the full demolition of a building or simultaneously with the removal of one or more stories of a building, except as provided in this subdivision and the rules of the department.

(2)  Prior to the issuance of a full demolition permit by the department of buildings, the owner of the building to be demolished shall submit to the department of buildings (i) certification, in a form to be provided by the rules of the department of environmental protection, that the building is free of asbestos containing material or, (ii) documentation that the commissioner of environmental protection has issued a variance from this requirement pursuant to subdivision (i) of this section and the rules of the department, subject to the additional conditions set forth in paragraph four of this subdivision.

(3)  Prior to the issuance of an alteration permit by the department of buildings to remove one or more stories of a building, the owner of the building shall submit certification  to the department of buildings (i) that  the stories to be removed are free of asbestos containing material and that no asbestos activities will be performed anywhere in the building simultaneously with the removal work authorized by such permit or (ii) that the commissioner of environmental protection has issued a variance from these requirements pursuant to subdivision (i) of this section and the rules of the department, subject to the additional conditions set forth in paragraph four of this subdivision.

(4) Any variance granted by the commissioner of environmental protection pursuant to subdivision (i) of this section relating to the full demolition of a building or the removal of stories in a building that would permit the simultaneous performance of asbestos activities and demolition work in the same building shall require the approval of the commissioner of buildings and the fire commissioner and shall specify appropriate safeguards as determined by such commissioners.  The department of environmental protection shall consult with the department of buildings, the fire department and representatives of affected industries to establish criteria for the issuance of variances that will provide for the safety of such operations.

(5)  This subdivision shall not apply to full demolitions or the removal of one or more stories performed as emergency work pursuant to article 215 of chapter 2 of title 28 of the administrative code or other law.

§2.   Article 106 of chapter 1 of title 28 of the administrative code of the city of New York, as added by a local law of the city of New York amending the administrative code of the city of New York relating to asbestos abatement as proposed in a local law of the city of New York amending the administrative code of the city of New York entitled, in relation to  asbestos abatement and repealing article 106 of chapter 1 of title 28 of the administrative code in relation thereto, which was introduced on May 21st 2009, is amended by adding new sections 28-106.1.1 and 28-106.1.2 to read as follows:

§28-106.1.1 Full demolition permit.  The commissioner shall not issue a full demolition permit unless the owner of the building provides certification in a form and manner to be provided in the rules of the department of environmental protection that (i) the building is free of asbestos containing material, or (ii) the commissioner of environmental protection, with the advice of the commissioner of buildings and the fire commissioner, has issued a variance from this requirement in accordance with subdivision (o) of section 24-146.1 of the administrative code and the rules of the department of environmental protection, subject to the requirement that demolition work will be performed only in parts of the building that are certified free of asbestos containing material. The full demolition permit shall be subject to such additional conditions as the department of buildings may require of the permittee based on the size and complexity of the demolition job.

Exception: This section 28-106.1.1 shall not apply to full demolition performed as emergency work pursuant to article 215 of chapter 2 of this title or other law.

§ 28-106.1.2 Alteration permit for the removal of one or more stories.  The commissioner shall not issue an alteration permit for the removal of one or more stories of a building unless the owner of the building provides certification in a form and manner to be provided in the rules of the department of environmental protection that (i) the stories to be removed are free of asbestos containing material and that no asbestos activities will be performed anywhere in the building simultaneously with the removal work authorized by such permit or (ii) the commissioner of environmental protection has issued a variance from these requirements in accordance with subdivision (o) of section 24-146.1 of the administrative code and the rules of the department of environmental protection, subject to the requirement that  work authorized by the alteration permit will be performed only in parts of the building that are certified free of asbestos containing material. The alteration permit shall be subject to such additional conditions as the department of buildings may require of the permittee based on the size and complexity of the work.

Exception: This section 28-106.1.2 shall not apply to removal of one or more stories performed as emergency work pursuant to article 215 of chapter 2 of this title or other law.

§3.   This local law shall take effect on the same date as a local law of the city of New York amending the administrative code of the city of New York entitled, in relation to  asbestos abatement and repealing article 106 of chapter 1 of title 28 of the administrative code in relation thereto, takes effect, except that prior to such date the fire commissioner, the commissioner of buildings and the commissioner of environmental protection may take any administrative action, including the promulgation of rules, that may be necessary for the timely implementation of this local law.

RCC

BKH

05/07/09

7:00 p.m.

LS # 6579