Staff:	Baaba K. Halm

Assistant Counsel to the Committee

Benjamin J. Goodman

Senior Legislative Policy Analyst

Ralph P. Hernandez

Legislative Financial Analyst

THE COUNCIL

REPORT OF THE INFRASTRUCTURE DIVISION

ROBERT NEWMAN, LEGISLATIVE DIRECTOR

COMMITTEE ON HOUSING AND BUILDINGS

Hon. Erik Martin Dilan, Chair

October 28, 2009

PROPOSED INT. NO. 874-A:	By:	Council Members Dickens, Comrie, Eugene, Gerson, Gonzalez, Liu, Arroyo, Gioia, Katz, Mark-Viverito, Baez, de Blasio, Recchia Jr., Gentile, Lappin, Mitchell, Sears, Reyna, Crowley, Sanders Jr., Vallone Jr., Ferreras, Jackson, Oddo and Ulrich

TITLE:	A Local Law to amend the administrative code of the city of New York, in relation to methane and radon gas vent piping, the approval of mechanical joint piping systems and the approval of non-code prescribed drainage vent systems.

ADMINISTRATIVE CODE:	Amends sections 202, 605.12.2, 605.15.3, 605.14.2,  605.22.1, 605.23, 705.2.1, 705.4.2, 705.9.2, 705.10.2, 705.12.2, 705.14.1, 901.1, 918.1, and 919.1 of the Plumbing Code of title 28 and adds a new section 901.1.1 to such Code.

BACKGROUND AND ANALYSIS:

On October 28, 2009, the Committee on Housing and Buildings, chaired by Council Member Erik Martin Dilan, will conduct a hearing on Proposed Int. No. 874-A, A Local Law to amend the Administrative Code of the City of New York, in relation to methane and radon gas vent piping, the approval of mechanical joint piping systems and the approval of non-code prescribed drainage vent systems. The Council is concerned about the release of methane and radon from underground areas into the atmosphere. The Committee first heard testimony on  Proposed Int. No. 874-A on October 15, 2009 from representatives of the Department of Buildings, labor representatives and other persons interested in this legislation.

Proposed Int. No. 874-A

Bill section one amends the definition of “VENT PIPE” in section 202 of the Plumbing Code of the City of New York (PC or Code) by including a reference to “Vent System (Methane and Radon)” to the definition and in bill section two defines “Vent System  (Methane and Radon)” as a pipe which allows methane or radon gas to pass from an underground area into the atmosphere in a safe way that does not allow the gases to harmfully affect the population nearby or to provide for recovery of the gases for commercial purposes.

Bill section three amends section 605.12.2 of the PC to only allow the use of mechanical joints when such joints are installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the Commissioner of Buildings (the Commissioner).

Bill section four amends section 605.14.2 of the PC to allow the use of mechanical joints in copper pipes only where such joints are installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the Commissioner.

Bill section five amends section 605.15.3 of the PC to allow the use of mechanical joints in the joints between copper or copper-alloy tubing or fittings where such joints are installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the Commissioner.

Bill section six amends section 605.22.1 of the PC to allow for the use of mechanical joints between stainless steel pipe and fittings only in cases where such joints are installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the Commissioner.

Bill section seven amends section 605.23 of the PC, to allow for the joints between different piping materials to be made with a mechanical joint of the compression or mechanical-sealing type where such joint is installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the Commissioner.

Bill section eight amends section 705.2.1 of the PC to allow for the mechanical joints on drainage pipes to be installed only in underground systems in accordance with the manufacturer’s instructions and in conformance with acceptance criteria established by the Commissioner.

Bill section nine amends section 705.4.2 of the PC to allow for the use of mechanical joints between the brass pipes or fittings on sanitary drainage piping only where such joints are installed in accordance with the manufacturer’s instructions and in conformance with acceptance criteria established by the Commissioner.

Bill section ten amends section 705.9.2 of the PC to allow for the use of mechanical joints between copper or copper-alloy pipe or fittings on sanitary drainage piping only where such joints are installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the Commissioner.

Bill section eleven amends section 705.10.2 of the PC to allow for the use of mechanical joints between copper or copper-alloy tubing or fittings on sanitary drainage piping only where such joints are installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the Commissioner.

Bill section twelve amends section 705.12.2 of the PC to allow for the use of mechanical joints on galvanized steel pipe or fittings on sanitary drainage piping only where such joints are installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the Commissioner.

Bill section thirteen amends section 705.14.1 of the PC to allow for the use of mechanical joints between PVC plastic pipe or fittings on sanitary drainage piping only where such joints are installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the Commissioner.

Bill section fourteen amends section 901.1 of the PC by adding a provision exempting vent systems for radon and methane gases from the general scope of Chapter 9 of the Plumbing Code. A new section 901.1.1 is also added, which requires the design and materials used in the installation of the methane and radon vent systems to be approved by the Commissioner of Buildings and must comply with all applicable rules of the Fire Department.

Bill section fifteen amends section 919.1 of the PC to require that a computer designed vent system be approved by the Commissioner to insure compliance with the minimum standards of the Plumbing Code.

Bill section sixteen provides that this law would become effective one hundred eighty days after  enactment.

## Amendments to Proposed Int. No. 874-A

## Technical changes were made through out the bill to correct inaccurate cross references to the Plumbing Code.

## Bill sections three through thirteen were amended to require the Department of Buildings to establish acceptance criteria for the installation of mechanical joints and recognizes that such joints must also be installed in accordance with the manufacturers’ instructions. These amendments are intended to clarify that the Department will not have to inspect every installation of such joints or approve manufacturers’ instructions related to such use but in addition to such manufacturers’ instructions DOB must identify appropriate uses of mechanical joints by establishing acceptance criteria.

## Former bill section fifteen, which amended section 918.1 of the Plumbing Code requiring computer-designed engineered vent systems to comply with the Administrative Code and to be approved by the Commissioner was removed as such process is already required under the Plumbing Code.

## Finally, the enactment clause was amended to provide that this local law would become effective one hundred eighty days after enactment rather than ninety days as previously provided.

Update

On Wednesday, October 28, 2009 the Committee adopted this legislation.

Accordingly, the Committee recommends its adoption.

Proposed Int. No. 874-A

By Council Members Dickens, Comrie, Eugene, Gerson, Gonzalez, Liu, Arroyo, Gioia, Katz, Mark-Viverito, Baez, de Blasio, Recchia Jr., Gentile, Lappin, Mitchell, Sears, Reyna, Crowley, Sander Jr., Vallone Jr., Ferreras, Jackson, Dilan, Fidler, Vacca, Oddo and Ulrich

..Title

A Local Law

To amend the administrative code of the city of New York, in relation to methane and radon gas vent piping, the approval of mechanical joint piping systems and the approval of non-code prescribed drainage vent systems.

..Body

Be it enacted by the Council as follows:

Section 1. Section 202 of the New York city plumbing code, as added by local law number 99 for the year 2005, is amended by amending the definition of “Vent Pipe” to read as follows:

VENT PIPE. See “Vent system” and “Vent System (Methane and Radon).” 	§2. Section 202 of the New York plumbing code, as added by local law number 99 for the year 2005, is amended by adding a definition of “Vent System (Methane and Radon)” to follow the definition of “Vent System” to read as follows:

VENT SYSTEM (Methane and Radon).  A pipe or pipes installed to provide for the passage of methane or radon gas from underground areas into the general atmosphere in an effort to avoid the hazardous effects of the gases on human beings or to provide the recovery of these gases for commercial purposes.

§3. Section 605.12.2 of section 605 of the New York city plumbing code, as added by local law number 99 for the year 2005, is amended to read as follows:

605.12.2 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the commissioner.

§4. Section 605.14.2 of section 605 of the New York city plumbing code, as added by local law number 99 for the year 2005, is amended to read as follows:

605.14.2 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the commissioner.

§5.  Section 605.15.3 of section 605 of the New York city plumbing code, as added by local law number 99 for the year 2005, is amended to read as follows:

605.15.3 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the commissioner.

§6. Section 605.22.1 of section 605 of the New York city plumbing code, as added by local law number 99 for the year 2005, is amended to read as follows:

605.22.1 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the commissioner.

§7. Section 605.23 of section 605 of the New York city plumbing code, as added by local law number 99 for the year 2005, is amended to read as follows:

605.23 Joints between different materials.  Joints between different piping materials shall be made with a mechanical joint of the compression or mechanical-sealing type, or as permitted in sections 605.23.1 and 605.23.3. Connectors or adaptors shall have an elastomeric seal conforming to ASTM D 1869 or ASTM F 477. Joints shall be installed in accordance with the manufacturer’s instructions and in conformance with acceptance criteria established by the commissioner.

§8. Section 705.2.1 of section 705 of the New York city plumbing code, as added by local law number 99 for the year 2005, is amended to read as follows:

705.2.1 Mechanical joints. Mechanical joints on drainage pipes shall be made with an elastomeric seal conforming to ASTM C 1173, ASTM D 3212 or CAN/CSA B602. Mechanical joints shall be installed only in underground systems unless otherwise approved. Joints shall be installed accordance with the manufacturer’s instructions and in conformance with acceptance criteria established by the commissioner.

§9. Section 705.4.2 of section 705 of the New York city, as added by local law number 99 for the year 2005, is amended to read as follows:

705.4.2 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the commissioner.

§10. Section 705.9.2 of section 705 of the New York city plumbing code, as added by local law number 99 for the year 2005, is amended to read as follows:

705.9.2 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the commissioner.

§11.  Section 705.10.2 of section 705 of the New York city plumbing code, as added by local law number 99 for the year 2005, is amended to read as follows:

705.10.2 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the commissioner.

§12. Section 705.12.2 of section 705 of the New York city plumbing code, as added by local law number 99 for the year 2005, is amended to read as follows:

705.12.2 Mechanical joints. Joints shall be made with an approved elastomeric seal.  Mechanical joints shall be installed in accordance with the manufacturers’ instructions and in conformance with acceptance criteria established by the commissioner.

§13. Section 705.14.1 of section 705 of the New York city plumbing code, as added by local law number 99 for the year 2005, is amended to read as follows:

705.14.1 Mechanical joints. Mechanical joints on drainage pipes shall be made with an elastomeric seal conforming to ASTM C 1173, ASTM D 3212 or CAN/CSA B602. Mechanical joints shall not be installed in above-ground systems unless otherwise approved. Joints shall be installed accordance with the manufacturer’s instructions and in conformance with acceptance criteria established by the commissioner.

§14. Section 901.1 of section 901 of the New York city plumbing code, as added by local law number 99 for the year 2005, is amended and a new section 901.1.1 is added to read as follows:

901.1 Scope. The provisions of this chapter shall govern the material, design, construction and installation of vent systems except for vent systems for methane and radon which shall be governed by this section.

901.1.1 Methane and radon venting. The design and materials used in the installation of the methane and radon vent systems shall be approved by the commissioner and shall comply with all applicable rules of the fire department.

§15.  Section 919.1 of section 919 of the New York city plumbing code, as added by local law number 99 for the year 2005, is amended to read as follows:

919.1 Design of vent system. The sizing, design and layout of the vent system shall be permitted to be determined by [approved] computer program design methods which shall be approved by the commissioner to insure compliance with the minimum standards of this code.

§16. This local law shall take effect one hundred eighty days after enactment.

10/20/2009 7:14 PM