Proposed Int. No. 874-A

By Council Members Dickens, Comrie, Eugene, Gerson, Gonzalez, Liu, Arroyo, Gioia, Katz, Mark-Viverito, Baez, de Blasio, Recchia Jr., Gentile, Lappin, Mitchell, Sears, Reyna., Crowley, Sander Jr., Vallone Jr., Ferreras, Jackson, Dilan, Fidler, Vacca, Oddo and Ulrich

..Title

A Local Law to amend the administrative code of the city of New York, in relation to methane and radon gas vent piping, the approval of mechanical joint piping systems and the approval of non-code prescribed drainage vent systems.

..Body

Be it enacted by the Council as follow:

Section 1. Section PC202 of the plumbing code of the city of New York is amended by amending the definition of “Vent Pipe” to read as follows:

VENT PIPE. See “Vent system” and “Vent System (Methane and Radon).” 	§2. Section PC202 of the plumbing code of the city of New York is amended by adding a definition of “Vent System (Methane and Radon)” to follow the definition of “Vent System” to read as follows:

VENT SYSTEM (Methane and Radon).  A pipe or pipes installed to provide for the passage of methane or radon gas from underground areas into the general atmosphere in an effort to avoid the hazardous effects of the gases on human beings or to provide the recovery of these gases for commercial purposes.

§3. Section 605.12.2 of section PC605 of the plumbing code of the city of New York is amended to read as follows:

605.12.2 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions when such instructions have been approved by the commissioner or in accordance with such other procedure which has received prior approval from the commissioner.

§4. Section 605.14.2 of section PC605 of the plumbing code of the city of New York is amended to read as follows:

605.14.2 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions when such instructions have been approved by the commissioner or in accordance with such other procedure which has received prior approval from the commissioner.

§5.  Section 605.15.3 of section PC605 of the plumbing code of the city of New York is amended to read as follows:

605.12.2 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions when such instructions have been approved by the commissioner or in accordance with such other procedure which has received prior approval from the commissioner.

§6. Section 605.22.1 of section PC605 of the plumbing code of the city of New York is amended to read as follows:

605.22.1 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions when such instructions have been approved by the commissioner or in accordance with such other procedure which has received prior approval from the commissioner.

§7. Section 605.23 of section PC605 of the plumbing code of the city of New York is amended to read as follows:

605.23 Joints between different piping materials shall be made with a mechanical joint of the compression or mechanical-sealing type, or as permitted in sections 605.23.1 and 605.23.3. Connectors or adaptors shall have an elastomeric seal conforming to ASTMD 1869 or ASTMF 477. Joints shall be installed in accordance with the manufacturer’s instructions when such instructions have been approved by the commissioner or in accordance with such other procedure which has received prior approval from the commissioner.

§8. Section 705.2.1 of section PC705 of the plumbing code of the city of New York is amended to read as follows:

705.2.1 Mechanical joints. Mechanical joints on drainage pipes shall be made with an elastomeric seal conforming to ASTMC 1173, ASTMD 3212 or CAN/CSAB602. Mechanical joints shall be installed only in underground systems unless otherwise approved. Joints shall be installed accordance with the manufacturer’s instructions when such instructions have been approved by the commissioner or in accordance with such other procedure which has received prior approval from the commissioner.

§9. Section 705.4.2 of section PC705 of the plumbing code of the city of New York is amended to read as follows:

705.4.2 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions when such instructions have been approved by the commissioner or in accordance with such other procedure which has received prior approval from the commissioner.

§10. Section 705.9.2 of section PC705 of the plumbing code of the city of New York is amended to read as follows:

705.9.2 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions when such instructions have been approved by the commissioner or in accordance with such other procedure which has received prior approval from the commissioner.

§11.  Section 705.10.2 of section PC705 of the plumbing code of the city of New York is amended to read as follows:

705.10.2 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions when such instructions have been approved by the commissioner or in accordance with such other procedure which has received prior approval from the commissioner.

§12. Section 705.12.2 of section PC705 of the plumbing code of the city of New York is amended to read as follows:

705.12.2 Mechanical joints. Mechanical joints shall be installed in accordance with the manufacturers’ instructions when such instructions have been approved by the commissioner or in accordance with such other procedure which has received prior approval from the commissioner.

§13. Section 705.14.1 of section PC705 of the plumbing code of the city of New York is amended to read as follows:

705.14.1 Mechanical joints. Mechanical joints on drainage pipes shall be made with an elastomeric seal conforming to ASTMC 1173, ASTMD 3212 or CAN/CSAB602. Mechanical joints shall be installed only in underground systems unless otherwise approved. Joints shall be installed accordance with the manufacturer’s instructions when such instructions have been approved by the commissioner or in accordance with such other procedure which has received prior approval from the commissioner.

§14. Section 901.1 of section PC901 of the plumbing code of the city of New York is amended and a new section 901.1.1 is added to read as follows:

901.1 Scope. The provisions of this chapter shall govern the material, design, construction and installation of vent systems except for vent systems for methane and radon which shall be governed by this section.

901.1.1 Methane and radon venting. The design and materials used in the installation of the methane and radon vent systems shall be approved by the commissioner and shall comply with all applicable rules of the fire department.

§15.  Section 918.1 of section PC918 of the plumbing code of the city of New York is amended to read as follows:

918.1 General. Engineered vent systems shall comply with this section and section 28-105 of this code and shall be approved by the commissioner.

§16.  Section 919.1 of section PC919 of the plumbing code of the city of New York is amended to read as follows:

919.1 Design of vent system. The sizing, design and layout of the vent system shall be permitted to be determined by [approved] computer program design methods which shall be approved by the commissioner to insure compliance with the minimum standards of this code.

§17. This local law shall take effect ninety days after enactment.

1-23-09